SAFETY DATA SHEET
Granite
American Granite Tile and Stone LLC
Effective Date: 4/24/2025

SECTION 1: Identification

Product Name: Granite (natural stone)
Synonyms: Natural Stone, Dimensional Stone
Manufacturer: American Granite Tile and Stone LLC
Address: 937 W. Miracle Mile
Phone: 520-834-4700
Emergency Contact: 520-834-4700
Recommended Use: Countertops, flooring, decorative stone
Restrictions on Use: Not intended for inhalation of dust

SECTION 2: Hazard Identification

Classification (OSHA 29 CFR 1910.1200):
Granite in its natural state is not hazardous, but dust generated during cutting, grinding, or polishing may contain respirable crystalline silica.

Signal Word: DANGER

Hazard Statements:

- H372: Causes damage to lungs through prolonged or repeated inhalation of dust (silica)

Precautionary Statements:

- P260: Do not breathe dust

- P264: Wash hands after handling

- P280: Wear protective gloves/eye protection/respiratory protection when cutting

- P501: Dispose of dust waste in accordance with local regulations

SECTION 3: Composition / Information on Ingredients

| Component | CAS Number | % by Weight |
| --- | --- | --- |
| Natural Granite (varies) | Mixture | >90% |
| Crystalline Silica (Quartz) | 14808-60-7 | 10–45% |

Note: Exact composition varies by quarry/source.

SECTION 4: First Aid Measures

Inhalation: Move to fresh air. Seek medical attention for symptoms of silicosis or breathing difficulty.
Skin Contact: Wash with soap and water.
Eye Contact: Rinse cautiously with water. Remove contact lenses if present and easy.
Ingestion: Not a typical route of exposure. Seek medical advice if a large quantity is ingested.

SECTION 5: Firefighting Measures

Suitable Extinguishing Media: Use extinguishing media appropriate for surrounding fire.
Specific Hazards: Granite is not flammable.
Protective Equipment: Wear standard PPE for surrounding fire conditions.

SECTION 6: Accidental Release Measures

Personal Precautions: Avoid dust formation. Use adequate ventilation.
Cleanup: Sweep up using methods that minimize dust generation. Wet methods preferred.
Environmental Precautions: Prevent dust from entering water systems.

SECTION 7: Handling and Storage

Handling: Avoid inhalation of dust. Use wet cutting or local exhaust ventilation.
Storage: Store in dry area. Use care when moving slabs due to weight.

SECTION 8: Exposure Controls / Personal Protection

Exposure Limits (OSHA PEL):

- Respirable Crystalline Silica (Quartz): 50 µg/m³ (8-hour TWA)

Engineering Controls: Use wet methods or local exhaust ventilation
PPE:

- Respiratory: N95 or higher during cutting

- Eye: Safety glasses or goggles

- Skin: Gloves and protective clothing

SECTION 9: Physical and Chemical Properties

- Appearance: Solid stone, various colors

- Odor: None

- pH: Not applicable

- Boiling Point: Not applicable

- Melting Point: >1200°C

- Solubility: Insoluble

- Density: ~2.6–2.9 g/cm³

SECTION 10: Stability and Reactivity

Reactivity: Not reactive
Stability: Stable under normal conditions
Hazardous Decomposition: None known
Incompatible Materials: Strong acids may etch surface

SECTION 11: Toxicological Information

- Inhalation: Long-term inhalation of dust containing crystalline silica may cause silicosis or lung cancer.

- Skin/Eyes: Mechanical irritation possible

- Carcinogenicity: Crystalline silica (in dust form) is classified as Group 1 (carcinogenic to humans) by IARC.

SECTION 12: Ecological Information

Not expected to be harmful to the environment. Not biodegradable. Inert solid material.

SECTION 13: Disposal Considerations

Dispose of in accordance with local, state, and federal regulations. Avoid dust dispersal.

SECTION 14: Transport Information

Not regulated as a hazardous material by DOT, IATA, or IMDG.

SECTION 15: Regulatory Information

- OSHA: Hazardous due to respirable crystalline silica

- TSCA: Components listed

- SARA 313: Not reportable

- California Prop 65: WARNING: This product can expose you to crystalline silica, known to the State of California to cause cancer.

SECTION 16: Other Information

Prepared by: American Granite Tile and Stone LLC
Date of Preparation/Revision: [Insert Date]
Disclaimer: The information provided is believed to be accurate but is not guaranteed. The user assumes all responsibility for use and handling.